Situation recognition as a step to an intelligent situation-aware crew assistant system

Quint Mouthaan    Patrick Ehlert   Leon Rothkrantz

Delft University of Technology, Mekelweg 4, 2628 CD, Delft, The Netherlands

Abstract

In this paper we will present a system that can recognize situations during a flight in real-time based on the data from an aircraft’s systems. The system uses Bayesian belief networks to calculate the probabilities of both the start and end of possible situations and from this distillates the most probable situation. The situation recognizer system is part of our test environment to create better human-machine interfaces in the cockpit.

# Introduction

## Problem statement

Anyone who has seen the cockpit of an F-16 aircraft knows that it is stuffed with control buttons, meters, and displays. The meters and displays provide the pilot with a wealth of information during a flight. Since the F-16 is capable of speeds of over 2000 Km/h, pilots have very little time to process the large amount of available information and make decisions. To help a pilot deal with information processing and decision-making, avoid information overload, and optimise flight performance, a Crew-Assistant System (CAS) or intelligent pilot-vehicle interface (PVI) has been proposed [1,2]. A typical CAS is shown in Figure 1. The idea is that the system presents relevant information to the pilot at the right moment and in the appropriate format, depending on the situation, the status of the aircraft, and the workload of the pilot. It is even possible that the CAS takes over (simple) tasks. Not only military pilots can benefit from such a system, it is useful for commercial pilots as well.

## The ICE project

The Intelligent Cockpit Environment (ICE) project is a project of the Knowledge Based Systems group of Delft University of Technology. The goal of the ICE project is to design, test, and evaluate computational

Figure 1:  A generic crew assistance system architecture

techniques that can be used in the development of intelligent situation-aware CASs. Using methods from artificial intelligence, ICE focuses primarily on the data fusion, data processing and reasoning part of these systems. Special issues addressed in the ICE project are situation recognition, mission or flight plan monitoring, pilot workload monitoring, and attack management [4].

In this paper we will discuss a system for real-time situation recognition that we developed, which is the most important subsystem in the situation assessment module shown in Figure 1. First, we will describe the design of the recognition system. Then we will discuss an example scenario that was used to test our system. Finally, we will draw some conclusions about the performance of the system.

# The design

## System architecture

The goal of our system is to derive the current situation in real-time from available aircraft data. The system receives information from the airplane’s sensors about the state of the airplane (e.g. the airspeed), the actions of the pilot (e.g. lowering the landing gear), and the environment (e.g. a missile that has been launched). Our system will use all this information to determine which of all possible situations is occurring. This list of possible situations is defined in an XML file. For every situation, the actions the pilot might perform and typical situation-related  events are defined. These events can either be changes in the state of the airplane or changes in the environment. During a flight the system will compare the information it receives from the sensors with the stored situations data and it will try to determine which situation is occurring. The architecture of the system is depicted in Figure 2. We will now explain in short the elements of the system.

Figure 2: The architecture of the situation recognition system.

The input module receives aircraft data such as airspeed, altitude, throttle, etc. from the flight simulator and converts this data to facts that are forwarded to the rule base.

The knowledge converter converts all the situations knowledge to IF-THEN rules and puts them in the rule base.

The flight plan interpreter converts the information in the flight plan to a number of rules that are put in the rule base. These are rules that predict which situations will occur in the near future.

The rule base contains all the rules that have been generated from the earlier described modules. When data from the flight simulator is added to the rule base in the form of a number of facts, some of the rules will fire and will generate probabilities concerning the start or end of a situation. These probabilities are passed to the overall controller.

The overall controller gets all probabilities from the rules in the rule base and some extra information about the situations. The overall controller combines the probabilities and calculates for every situation the probability that it has started and the probability that it has ended. Based on the resulting probabilities, the overall controller will draw a conclusion about the situation that is most likely to be the current one. Calculating the probabilities will be done using Bayesian belief networks, which will be discussed in the next section. More information about the recognition system can be found in [5].

## Probability inference using Bayesian belief networks

We want to calculate two probabilities for every situation: the probability that the situation has started and that it has ended. Therefore we need two Bayesian belief networks. The two networks are explained in the following sections.

### The start probability calculator

Figure 3 shows the Bayesian belief network that is used to calculate the probability that a situation has started.

Figure 3: The Bayesian belief network that calculates the probability that a situation has started.

The start conditions for a situation are conditions that must be satisfied before a situation can possibly have started. When the start conditions are satisfied the probability of the start constraints that is specified in the situations knowledge will be the output of this node.

The action probabilities are passed to the Bayesian belief network by the rules in the rule base. These probabilities all contribute to the probability that the situation is occurring.

The additional rules are rules that fire when the state of the aircraft changes or when a specific event happens. When they fire they can generate a probability that a situation has started or ended.

The probability calculator combines the probabilities of the nodes that have been described above using the noisy-OR model.

The previous situation influences the start probability of a situation because the probability that a situation is occurring should rise when the probability that one of the previous situations has ended becomes higher.

Based on this Bayesian belief network the probability that a situation has started and is occurring can be calculated with the following formula:

Pstart=Pcond∗Pend∗PsPcond=0 if Psc=01 if Psc>0Ps=1−((1−Psc)∗i=1n(1−Pcai)∗j=1n(1−Pcrj))

In this formula is Psc the probability of the start conditions, Pend is the probability that one of the previous situations has ended, Pcai is the probability of the i-th action that should be performed during the situation and Pcrj is the probability of the j-th event or change in state that can occur during the situation.

### The end probability calculator

In Figure 4 the Bayesian belief network to calculate the probability that the situation has ended is shown. In this Bayesian belief network we see a lot of the same nodes as in the belief network for the start of the situation. The nodes that are different are discussed below.

The time window for a situation is the maximum duration of that situation. If the start of a situation has been detected the probability that it has ended should become larger when a bigger part of the time window has elapsed. This is implemented with the time window node.

The situation started node produces a 1 if the situation has started and a 0 if the situation has not started yet. This node is necessary because we only want to calculate the probability that the situation has ended if the situation has started.

Figure 4: The Bayesian belief network that calculates the probability that a situation has ended.

The probability that the situation is not occurring can be calculated with the following formula:

Pend=1−((1−Psc)∗(1−Pt)∗i=1n(1−Pcri))

# Experiments and results

The system was tested using the Microsoft Flight Simulator. Experiments were performed with an F-16 and with a Cessna. The results of one of our experiments are presented in Table 1. The first column contains the names of the situations that occurred and/or were detected. The second column contains the times at which we considered the situations to be started. The third column contains the times at which the situations were detected by the system. The times are given in seconds from the moment the program was started. The particular mission of Table 1 consisted of an attack on a ground target in an F-16. During the flight to the target the pilot had to check his course twice. After the attack had been performed the pilot returned to the airbase and landed the airplane.

Table 1: The test results

| Situation | Time started (s) | Time detected (s) |
| --- | --- | --- |
| Startup | 0 | 0 |
| Taxiing to runway | 10 | 12 |
| Taking off | 14 | 15 |
| Normal flight | 43 | 34 |
| Navigating | 83 | 83 |
| Normal flight | 91 | 91 |
| Navigating | 123 | 123 |
| Normal flight | 128 | 128 |
| Visual attack | 186 | 193 |
| Normal flight | 221 | 221 |
| Landing | 361 | 366 |
| Aborting a landing | - | 410 |
| Taxiing from runway | 409 | 410 |
| Shutdown | 427 | 427 |

From the table it is clear that the program is able to recognize most situations in a matter of seconds. The reason that it sometimes takes the system some time (5 seconds or more) to detect some of the situations is mainly because the start and end points of those situations are very vague. For example, we have considered the situation Normal flight to start at the moment at which the pilot levelled the airplane, while the program considered the situation to be started when the pilot reached a particular minimum altitude. This could be solved by adjusting the situations knowledge to change the definition of the start of a landing of the system.

The landing was a normal landing and was detected, but as is shown in the table the program thought for a moment that the landing was being aborted. This happened when the program knew that the landing had been finished and looked for the situation with the highest start probability. This turned out to be the situation Aborting landing. This is because at some point the pilot had moved the throttle to the maximum. This action was still in the memory of the program when the landing ended. The fact that the landing gear was raised at the start of the landing was also still in the memory of the program. Because of this the probability that the landing was being aborted was high and the situation Aborting landing became the current one once the landing had finished. However as soon as that happened the program realized that the airplane was actually taxiing and it corrected the mistake immediately.

# Conclusions and future work

We have created a system that can recognize the current situation during a flight with an F16 and with a Cessna. The system is based on a probabilistic model. A rule base was created that compared data from a flight simulator with the situations knowledge defined in an XML file. The rules in the rule base generate a number of probabilities that are combined using Bayesian belief networks to calculate the probabilities that the situations are occurring. Based on these probabilities a conclusion is drawn about the situation that is most likely to be occurring.

The program was tested on a number of test scenarios. After analysing the results we can conclude that the system performs really well. It makes few mistakes and is able to correct them. Furthermore it is able to come to a conclusion about the current situation in real-time.

We can conclude that the program would perform even better if it would be extended to a causal model. This causal model could take into account the relations between actions and events.

Future work will consist of adding causality to make the system more reliable. Once that has been done the system can be used in conjunction with a workload assessment module to create a more complete situation awareness module. This situation awareness module can be used in an intelligent cockpit system that monitors and supports the pilot during a flight.

References

[1]	Banks, S.B and Lizza, C.S. (1991) “Pilot’s Associate: a cooperative, knowledge-based system application”, in IEEE Intelligent Systems, Vol. 6, No. 3, pp. 18-29, June 1991.

[2]	Onken, R. (1997) “The cockpit assistant system CASSY as an on-board player in the ATM environment”, paper presented at 1st USA/Europe Air Traffic Management R&D Seminar, Saclay, France, June 1997.

[3] 	Endsley, M.R. (1995) “Towards a theory of situation awareness in dynamic systems”, in Human Factors, Vol. 37, No. 1, pp 32-64.

[4] 	http://www.kbs.twi.tudelft.nl/Research/Projects/ICE/

[5] Mouthaan, Q.M. (2003) “Towards an intelligent cockpit environment: a probabilistic approach to situation recognition in an F-16”,  MSc. thesis, Knowledge Based Systems,  Delft University of Technology, The Netherlands.