# Interview with an F16 pilot.

Subject: Gideon Reisel

Interviewed by: Quint Mouthaan

Date: 4 February 2003

Time: 9.00 – 14.00

Location: Delft Technical University

# Introduction

This interview was conducted in the course of my graduation project on the Delft University of Technology. The goal of my graduation project is to design and implement a system that can detect the situation an F16 fighter pilot is in by monitoring the actions of the pilot and the state of the airplane. The first step of this project was collecting knowledge about the domain, in other words, I had to find out how an F16 pilot should react in different situations. I eventually found the information I needed in a manual of the US Air Force and the manual for the flight simulator game Falcon 4. To verify this information I interviewed Gideon Reisel, a former F16 pilot for the Dutch Royal Air Force. The goal of the interview was to verify and complement the information I found in the manuals.  I also had a lot of questions for which I couldn’t find the answers in the manuals but for which I hoped mr. Reisel would know the answers.

In this document the results of the interview have been written. First the questions that have been asked and the answers that have been given are written down, then in the next paragraph the information that has been given outside the scope of the questions is given. Finally of the mr. Reisel was asked to give an indication of the time windows of the situations and these have been put in the last paragraph. Then some conclusions are drawn about the value of the interview for my graduation project.

# The interview

## The agenda

To get as much information out of the interview as possible I wanted to let mr. Reisel fly a couple of missions with the Falcon 4 flight simulator while observing his actions. After that I asked the questions that had yet remained unanswered. The agenda looked like this:

9.00 – 10.00: Introduction to the project.

10.00 – 12.30: Flying and monitoring missions with Falcon 4.

12.30 – 13.00: Lunch

13.00 – 14.00: Answering remaining questions.

## Questions and answers

To describe what information was exchanged during the interview the most interesting questions with the answers to them are given here.

Taxiing:

Q: There are three different kinds of brakes: speed brakes, wheel brakes and parking brakes. What exactly is the difference between these and when are they used?

A: Speed brakes are brakes which decrease speed while in the air as well as on the ground. They are located between the horizontal stabilizer and the rudder. The wheel brakes can be controlled with pedals at the pilot’s feet. They have anti skid, so that the wheels can not block. Therefore there is no maximum speed above which the wheel brakes may not be used anymore. The parking brakes may only be used when the airplane is standing completely still, because these really block the wheels.

Q: If the pilot is taxiing and he is cleared for takeoff when he arrives at the runway is he allowed to continue without holding short?

A: Yes, the pilot can immediately enter the runway and takeoff without delays.

Q: When is NWS switched off?

A: As soon as the IAS indicator shows movement [this is at approximately 60 kts] the rudder is effective and the NWS becomes too sensitive and then the rudder will be the primary means to steer. As the nosewheel and the rudder are both controlled by the rudder pedals at the pilot’s feet, the changeover is easy.

Q: Are the fuel tanks always completely full at takeoff?

A: The inner fuel tanks usually are, the extra outer fuel tanks may not be full. An exception might be an airplane that is giving a demonstration at an air show. Then the inner fuel tanks might not be full either.

Taking off:

Q: Is the afterburner often used at taking off?

A: Usually the afterburner is not used when taking off. Only when the stopping distance becomes a factor the take-off roll may be shortened by using the afterburner.

Q: At what moment must the pilot decide at the latest to abort a take off?

A: When the pilot reaches the calculated Refusal Speed. That speed is determined during the briefing and depends on the weight of the aircraft, the length of the runway etc.

Q: Is the hook usually lowered if a takeoff is aborted?

A: Yes, but the hook has proven to be unreliable, so it is still used, but mostly the parachute is deployed first.

Q: Could it happen that the pilot tries to take off again after aborting a takeoff?

A: Yes, if the takeoff was aborted due to something that doesn’t affect the functionality of the airplane, the pilot might taxi to the runway and take off again.  The only limiting factor may be the temperature of the brakes. They need to cool down before raising them into the wheel wells.

Q: What is the maximum gear speed?

A: Around 300 knots.

Navigating:

Q: What is the most important instrument that is used to stay on course?

A: The route between two steerpoints is called a track. When you follow the INS steering to the next waypoint you will fly the correct track. The heading indicator can not be used to stay on this track, because the heading depends on the wind. The nose has to be turned into the wind to fly a straight path. Therefore visual checks of the environment are very important for navigating to the steerpoint and for evaluating the performance of the Navigation Systems.

Dogfight:

Q: Might some missiles be fired without a radar?

A: Most missiles can be fired without a radar lock. It is harder though because the distance to the target is not known. It might be too far away or too close. Furthermore the radar will calculate all the data compiled in a graphic display on the HUD called a DLZ or dynamic launch zone.

A-G attack:

Q: At what moment is the Master Arm switch set to ARM?

A: The moment enemy territory is entered.

Q: Which master mode is switched to after an attack?

A: It is usually switched to A-A mode. This gives almost the same info as NAV with the added advantage that the pilot is ready for other fighters.

Q: When a LGB attack is performed by two fighters of which one has to lock the target with his laser, what is the task of the supporting fighter?

A: The supporting fighter will circle at a safe distance around the target while he keeps his laser on the target. The laser arm switch is set to ARM at the latest moment possible because otherwise it emits a signal that might be picked up be enemy units. The fighter with the LGB’s will toss his bombs in a cone above the target that assures the weapon(s) can “glide” towards the laser signal.

Q: What does FTT (Fixed Target Track) mean exactly?

A: When a target is designated twice it is in FTT, which results in some extra info about the target being displayed.

Taking evasive action:

Q: Should the master lights be switched off when a missile launch is detected?

A: In wartime the lights will always be switched off.

Deep stall recovery:

Q: What does the speed meter display in a deep stall?

A: Airflow is severely disrupted around an airplane in deep stall, therefore are indications on any instrument are highly unreliable. The airspeed should be bigger than zero. It’s hard to say exactly, but it could be plus minus 80 knots.

Air refueling:

Q: How is the TACAN set for air refueling?

A: The TACAN can be used to find the tanker, when the TACAN channel of the tanker is known. The F16 pilot can then set his TACAN channel to a value that is a set distance from the channel of the tanker. This way the pilot can get a range on the tanker.

Landing:

Q: What is the airplane state before landing? Should the radar be switched off?

A: All weapon systems have been shut down on exiting the enemy territory, so they don’t have to be shut off anymore. Radar does NOT have to be shut off before landing. It should be shut off before taxiing because the radar waves might mot be healthy for the ground crew.

Q: How often does the pilot use ILS when landing?

A: Landing on instruments is only done when the visibility is very poor. Given a choice a pilot will always land the plane by visual means.

Q: Is the afterburner used when a landing is aborted?

A: Usually not.

Shut down:

Q: What is the shut down sequence? And is it always done by the pilot or are some things done by the ground crew?

A: The shut down sequence is about the same as the startup sequence. It is done by the pilot, only some things on the outside of the airplane are taken care of by the ground crew.

Instruments:

Q: What’s the difference between the positions START1 and START2 on the JFS switch?

A: The JFS is a starter for the engine. It has two bottles. When the JFS is in START1 only one bottle is used to start the engine. In START2 both bottles are used.

Q: What values does the Master Arm switch have? In the RC’s there is a value AUTO?

A: The value AUTO doesn’t exist. The Master Arm has the values: ARM, SIM and OFF.

Q: What do the VRP and VIP modes mean?

A: The VRP mode is a kind of offset mode. The target is known relative to another point. The radar cursors are slewed towards the target. In the VIP mode, the pilot notifies the system when he flies over the IP. Then the computer knows the position of the target relative to that IP.

Q: How does the IFF work?

A: The IFF broadcasts information about the airplane. It should be switched off in enemy territory and before air refueling.

Q: Can the trim be adjusted on the trim panel, or is that only a display of the trim settings?

A: Trim is set automatically in the F16. It can be manually adjusted though on the trim panel but also with one of the buttons on the stick.

Q: What buttons are used to slew the radar cursors?

A: There is a small joystick on the throttle.

Q: What is the functionality of the ATT/FPM switch and must it be on for performing an A-G attack?

A: If the ATT/FPM switch is set to on, the FPM adjusts itself for the crosswind. This can be annoying when the crosswind is very strong, because the FPM might go out of the HUD. It is important that the switch is on for an attack.

Visual checks:

Q: Is it necessary to check the climbing rate during air refueling or is it enough to look at the tanker?

A: The most important variables during the approach to the tanker are the distance and overtake values displayed in the HUD. When the pilot gets closer the visual cues will become more important and the attention of the pilot will be directed mainly at the tanker.

Q: Is the RPM gauge often checked?

A: No not really, the FTIT is checked more often, because that gives an indication of the temperature of the engine. The RPM is variable and depends on the outside atmospheric conditions.

Q: Does an F16 pilot use his peripheral vision to check the instruments in the cockpit?

A: No, if an F16 pilot wants to check something he looks directly at it.

## Extra information

Next to the answers of the questions a lot of information was given by mr. Reisel outside the scope of the questions which was still very interesting. Some of that information is given in this paragraph.

- The most crucial information for which the presentation could be improved in the F16 is the airspeed. It would be very convenient for a pilot to get a regular airspeed update or warning. The airspeed is very important for the energy of the airplane and it would help a lot if “bitching betty” would shout the airspeed in a dogfight or when the airspeed becomes low in a non landing situation.

- The EPU has three positions: OFF, AUTO and ON. When it is set in the AUTO position it will turn on automatically when the engine quits, when hydraulic pressure depletes or when the main engine driven generator fails.

- An evasive tactic that is often used for bullet shooting anti air guns is to change dimension every few seconds. This makes it impossible for the bullet shooter to predict the next position of the fighter to shoot at.

- In commercial airplanes the crucial altitudes are announced by a voice during the last part of landing which allows the pilot to concentrate on the landing better and to get a good feel for the “rate of descent” and trends.

- When the airplane gets in the dangerzone of getting into a deep stall, a loud horn will sound to give the pilot warning. It is said that the pilot is then in the Dog House.

- The standard scanning pattern for airplanes with a HUD includes the attitude from the HUD instead of the attitude of the AID. This is because the attitude from the HUD compensates for airspeed. At low airspeed the nose has to be raised to keep flying level, In that case the AID indicates a higher attitude, but the attitude in the HUD will compensate for the lower airspeed and stay the same. For the airspeed and the altitude the indicators in the cockpit are more often used than the indicators in the HUD, because pilots find it easier to discover a trend when looking at a clock. They get a better feeling of the variations in the altitude and airspeed from the clocks than from the digital numbers in the HUD. What could be useful is an arrow that indicates in which direction the airspeed or altitude is expected to go, this is already used in some commercial airplanes.

Furthermore mr. Reisel gave some information about the switches and buttons that are located on the throttle and the stick:

Throttle:

- A joystick for slewing radar cursors.

- A switch for the speed brakes.

- A switch for the dogfight or missile override modes.

- A button to control the radar tilt angle.

- And a knob to switch the radio between UHF and VHF.

Stick:

- A trim button.

- The NWS button.

- The gun trigger.

- The pickle button.

- The designate / RTS (Return To Search) button.

## The time windows

For all the situations mr. Reisel gave an indication of how long they would take on average:

Startup: 4 – 10 minutes.

Pilots that are on standby and must be in the air ASAP, a plane can be made ready in advance and then the pilot can be ready in 4 minutes. Normally the startup sequence will take about 10 minutes.

Taxiing: 3-5 minutes.

The time needed for taxiing depends on the distance to the runway or hangar of course.

Taking off: 1 minute.

Dogfight: 2 – 30 minutes.

In wartime a dogfight might last as short as 2 minutes, provided that the F16 doesn’t fight another F16, this is because of the superior qualities of the F16 for dogfighting. In peacetime during training a dogfight between two F16’s might last as long as 30 minutes, but normally in wartime, a dogfight will not take that long, because there is no time for that.

A-G attack: 2 –3 minutes.

This is the time it takes to get from the IP to the target.

Refueling: 5 minutes.

Landing: 2 minutes.

# Conclusions

After the interview it was clear that the knowledge that was collected was quite accurate. A number of corrections of and additions to the knowledge have been made due to the interview. It can also be concluded that Falcon 4 is a very realistic flight simulator. The only shortcome is that the situational awareness of the pilot is lower because of the two dimensional representation of the environment.

So in the end the interview turned out to be very important for my research project, not only because some assumptions that had been made turned out to be incorrect and which can now be corrected, but also because the collected knowledge has now been validated.