| Intelligent Agents in an Adaptive Cockpit Environment (Preliminary report) |  |
| --- | --- |
| Ehlert, Patrick Rothkrantz, Leon Research Report DKE01-01 Version 0.2, October 2001 Mediamatics / Data and Knowledge Engineering group Department of Information Technology and Systems Delft University of Technology, the Netherlands |  |

Ehlert, Patrick A.M.  (P.A.M.Ehlert@its.tudelft.nl)

Rothkrantz, Leon J.M.

“Intelligent Agents in an Adaptive Cockpit Environment” (Preliminary report)

Research Report  DKE01-01

Mediamatics / Data and Knowledge Engineering group

| Keywords: | Intelligent agents, adaptive cockpit environment, adaptive interfaces, workload assessment, situation awareness, artificial intelligence |
| --- | --- |

# Preface

This report is part of a project called Adaptive Cockpit Environment (ACE). It describes some theory and ideas about the use of intelligent agents in such an environment.

The first chapter briefly outlines the ACE project and its objectives. The next three chapters deal with some theory about intelligent agents. The idea is that these chapters provide some background about agents in general and about possible techniques that can be used in the ACE project. Chapter 2 explains what agents are and what properties they generally possess. Chapter 3 shows the advantages of using more than one agent in a multi-agent system.  In chapter 4 some techniques that are used to model and implement agents are described. Finally, in chapter 5 we propose to use four specialized agents that function as the logic and reasoning part of the ACE system.

# Summary

An agent is a computerized entity that can act on behalf of someone to carry out a particular task that has been assigned to it. Multiple agents can interact with each other to accomplish a task, usually through cooperation and coordination of their actions.

In the ACE project an adaptive cockpit interface should be constructed that adapts depending on the workload and status of a fighter pilot. We propose to use four agents that form the logic part of the ACE system and that reason about possible adaptation actions.

Table of Contents

Preface	I

Summary	III

Chapter 1: Introduction	1

1.1 Problem Setting	1

1.2 Project description	1

1.2.1 Project activities	2

1.2.2 Partners	2

Chapter 2: Intelligent agents	3

2.1 What exactly is an agent?	3

2.1.1 Agent definition	3

2.1.2 Agent properties	4

2.2 Agent types and classification	5

Deliberative agents	5

Reactive agents	5

Interface (or personal) agents	5

Heterogeneous (or hybrid) agents	6

2.3 Agent applications	7

2.4 Advantages and disadvantages	8

Chapter 3: Multi-agent systems	9

3.1 Introduction	9

3.1.1 Why use multi-agent systems?	9

3.2 Interaction, cooperation and coordination	9

3.3 Communication	11

Knowledge Query and Manipulation Language (KQML)	11

FIPA’s Agent Communication Language (ACL)	12

Knowledge Interchange Format (KIF)	12

Chapter 4: Building agents	14

4.1 The construction process	14

4.2 Knowledge representation and reasoning	14

Production systems	15

Fuzzy systems	15

Bayesian networks	15

Semantic networks	16

Frames	16

4.3 The knowledge base	16

4.4 Programming languages	17

4.5 Architectures	17

Blackboard-based architectures	17

The subsumption architecture	18

ARCHON	18

The Belief-Desire-Intention architecture	19

Chapter 5: Agents in the ACE System	20

5.1 Description of the ACE system	20

5.2 Sensors and input processing	21

5.2.1 Situation Assessment and Recognizer (SAR)	21

5.2.2 Workload and situation Awareness Module (WAM) 	22

5.2.3 Uncertainty of measurements	22

5.3 Reference component	22

5.4 Agents	22

The mission agent	22

The environment agent	22

The plane agent	22

The pilot agent	23

5.5 Output	23

Bibliography	26

# Introduction

In this chapter, the problem background (section 1.1) is outlined and a description of the ACE project (section 1.2) is given.

## Problem Setting

Due to the recent technological advances in aircraft performance and weapons capabilities, the time available to a pilot in a military aircraft has reduced significantly. Also, the amount of information available to a pilot and the complexity of the contents have increased. The pilot needs to assess a situation and make decisions in a split of a second. For this situation awareness (SA) is important, which means that the pilot needs to perceive and understand his situation and be able to predict the outcome. Having a high level of SA is seen as one of the most critical aspects for achieving successful performance in aviation [Endsley 1999a]. Also, many human errors in aviation are caused by lack of SA [Endsley 1999b].

An adaptive pilot-plane interface can improve the flow of information between the pilot and the aircraft in such a way that the pilot’s SA is improved and his mental workload is reduced. A multimodal interface can take advantage of a human’s capacity for parallel processing via multiple sensory modalities such as vision, sound and feeling. As a result the survivability of the pilot and plane and the effectiveness of the mission will be improved.

## Project description

As is also described in the ACE technical proposal, the goal of the ACE project is “the definition and evaluation of a prototype adaptable interface technique to identify the specific automation requirements and practical utility of this innovation in a military cockpit” [NLR 2000]. The idea is that certain features in the pilot-plane interface can be adapted or automated depending on the workload and status of the fighter pilot. Physiological measurements are taken to determine this workload and information from the aircraft system is used to determine the current situation. The high-level information supplied by the workload and situation assessment modules is then used by one or more agents (logical component) to determine the content, format and modality of the display in the cockpit. A picture of the ACE system is shown below.

Figure 1: simplified model of the ACE system and its environment

### Project activities

First a prototype system needs to be built that uses real-time techniques for determining the workload and SA of the pilot. At the same time a dynamic situation assessment component needs to be defined that can determine the current status and goals of the mission. Both components will use background knowledge stored in a reference module. Also the adaptive logic needs to be defined that is responsible for the actual content and modality of the information provided to the pilot. Finally, the system should be evaluated for its capabilities and limitations. The end result of the project should not necessarily be a fully functional system that can be implemented in real aircraft, but the main intent of the project is to gain experience on the usefulness of adaptive automation for fighter pilots.

### Partners

The ACE project is funded by the Dutch Ministry of Defense and monitored by the Royal Dutch Airforce. The active parties working on the project are:

- Military Academy in Brno, Czech Republic

  - Department of Technical Cybernetics and Military Robotics

- Delft University of Technology, the Netherlands

  - Department of Information Technology and Systems / Mediamatics group

- National Aerospace Laboratory, the Netherlands

  - Human Factors Department

# Intelligent agents

The term ‘agent’ has become a buzzword in the computer industry, especially on the Internet, and many software programs are given the name ‘agent’. In section 2.1 we will discuss what agents really are. Section 2.2 describes the most commonly found agent types and two distinct agent classifications are presented. A short overview of several applications is given in section 2.3. The chapter is closed with a discussion about the advantages and disadvantages of agents in section 2.4.

## What exactly is an agent?

An agent is a computerized entity that can act on behalf of someone to carry out a particular task that has been assigned to it. Obviously, we would like to use agents with as little effort as possible and since it is tedious to spell out every detail, we would like our agents to be able to understand what we mean from what we tell them in a natural fashion. Agents can only do this if they ‘know’ something about the context of the request. A good agent would not only need to have this particular expertise, but would also take into account the peculiarities of the user and the specific situation. In addition, we expect agents to be able to communicate with other agents and cooperate with them, and perhaps move from place to place in doing so.

Most agents that exist today are sophisticated software programs and only ‘live’ in cyberspace, but some agents exist in the physical world, for example a robot.

### Agent definition

As with many concepts in the field of artificial intelligence the term ‘agent’ is difficult to define. There are probably just as many definitions as there are researchers working on the subject and the fact that the word ‘agent’ is used (and misused) to refer to a wide range of computational entities, does not help either. Several definitions of well-respected researchers working on agents are:

“An intelligent agent is a computer system situated in some environment, and that is capable of flexible autonomous action in this environment in order to meet its design objectives.” [Jennings and Wooldridge 1998]

“An agent is an entity whose state is viewed as consisting of mental components such as beliefs, capabilities, choices and commitment.” [Shoham 1997]

“An agent is a program that is, to some degree capable of initiating actions, forming its own goals, constructing plans of action, communicating with other agents, and responding appropriately to events –all without being directly controlled by a human. (…) The second meaning of agent is connected with what is portrayed to the user. Here, ‘agent’ is used to describe programs which appear to have the characteristics of an animate being, often a human.” [Bradshaw 1997]

“When we really have to, we define an agent as referring to a component of software and/or hardware which is capable of acting exactingly in order to accomplish tasks on behalf of its user. Given a choice, we would rather say it is an umbrella term, meta-term or class, which covers a range of other more specific agent types.” [Nwana 1996]

Most definitions refer to several aspects commonly found in agents: autonomy, adaptivity and the ability to do a certain task. It is very hard to tell what qualities a program or entity must have to be classified as an agent. Shoham [1997] probably phrased it best by stating “what makes any hardware or software component an agent is precisely the fact that one has chosen to analyze and control it in these mental terms”. With this Shoham means that a program is an agent if one has decided to look at it as an agent.

Not only are there many different definitions of agents, researchers in the field also use several names and synonyms to distinguish different kinds of agents. Some examples are personal agents, personal assistants, autonomous agents, bots, softbots, robots, spiders, and crawlers. Generally speaking, agents that operate in the real world are called robots, and agents operating on the Internet are referred to as bots, spiders or crawlers. Agents that help a user to perform a particular task within a program or act on behalf of the user are called personal agents or personal assistants.

Comparisons have been made between an agent and an object in object-oriented programming languages. The main difference between the two is that an agent is considered to be a higher level concept. The actual implementation details of an agent can be hidden and neglected. They are much more sophisticated than objects in the sense that they are autonomous and are thought of as intelligent.

### Agent properties

Another way of defining an agent is by describing its properties. There are several characteristics that the ideal agent must possess in order to be truly intelligent [Jennings and Wooldridge 1998], [Nwana 1996]:

- Responsive (or reactive): an agent must be able to perceive its environment and be capable of performing actions based on its perceptions.

- Autonomous: the ability to perform actions without the constant guidance from the user. The agent must determine how a given problem is solved best and act on its own initiative.

- Adaptive: although not all agents are capable of adapting their behavior, adaptation is generally regarded as an important characteristic for intelligent agents. The agent should learn the user’s preferences and be able to improve from experience.

- Social: agents should be able to interact and cooperate with other agents or humans in order to carry out complex tasks they cannot handle alone and help others with their activities.

- Communicative: agents should be capable of communicating with other agents or humans using an expressive communication language. Ideally, this language should resemble human-like speech acts.

Several other, but generally regarded as less important, properties are [Jennings and Wooldridge 1998], [Nwana 1996], [Russell and Norvig 1995]:

- Mobility: the ability of an agent to move from one system to another to access remote resources or meet other agents.

- Personality: the capability of showing a ‘believable’ human-like character such as emotion.

- Inferential capability: an agent can act on abstract task specifications using prior knowledge of general goals. This goes beyond the given information and the agent may have explicit models of itself, the user, the situation and/or other agents.

- Proactive: agents should not simply act in response to their environment, but be able to perform opportunistic, goal-directed behavior and take the initiative where appropriate. Instead of waiting to be told what to do next, they should make suggestions to the user.

- Individual world view: every agent has its own model of the external world.

- Reproduction: if agents are able to reproduce themselves, they can evolve and adapt to changing situations, increasing their survival skills. This is one of the main research topics of artificial life.

Most of today’s software agents are designed for special-purpose applications and no agent can do very much of what is outlined above. Although several agents on the Internet might be able to pass the Turing test, the general consensus in the artificial intelligence community is that really intelligent agents have not yet been created.

## Agent types and classification

Different types of agents have been created based on the different properties that an agent should possess, for example deliberative agents, reactive agents, mobile (internet) agents, and interface agents. We will limit our discussion here to static (not mobile) agent types.

#### Deliberative agents

A deliberative or cognitive agent is an agent that possesses an explicitly represented symbolic model of the world and makes decisions via a symbolic reasoning process. The agent knows how to perform actions and can predict their outcome based on its knowledge about the dynamics of the world and how it evolves.

Deliberative agents are said to be intentional, i.e. they have goals and explicit plans to reach those goals. They can memorize situations, analyze them, foresee possible reactions of their actions and use this to decide what actions to perform. They use high-level instructions and can often negotiate with other agents to resolve conflicts. The disadvantage of deliberative agents is that their reasoning process takes quite some time, so they cannot be used for problems that need a real-time response. On the other hand, their intelligence and reasoning is often superior to that of other agent types.

#### Reactive agents

Reactive agents, also called reflex or behavior-based agents, react by stimulus-response rules to the current state of the environment perceived through their sensors. They have no representation or symbolic model of their environment and are incapable of foreseeing what is going to happen. The main advantage of reactive agents is that they are robust and have a fast response time, but the fact that purely reactive agents do not have any memory is a severe limitation. Also, the simplicity and functionality of their behavior is a sign of the intelligence of its designers, rather than of the agent. On the other hand, very complex patterns of behaviors can emerge from the interaction of multiple reactive agents.

Deliberative and reactive agents are two extremes, but many agents can be classified somewhere in between, using a combination of both approaches (see also Figure 2).

Figure 2: Part of the control system spectrum from [Arkin 1998]

#### Interface (or personal) agents

Interface agents cooperate with the user to accomplish a task and function as a personal assistant. Despite the name, the interface agent is usually not the interface between the user and a computer program. Instead, it observes the interaction between the user and the program from the sideline, learns from it, and interacts with both the user and the program.

Some interface agents are designed to train or teach the user, others can help multiple users to work together. Most interface agents are capable of learning and adapting to the interests, habits and preferences of the user. Pattie Maes, who has designed an e-mail assistant, has identified four different ways of learning for interface agents [Maes 1994]:

- Observing and imitating the user.

- Receiving positive or negative feedback from the user based on the agent’s performed actions.

- Receiving explicit instructions from the user.

- Asking other agents (with the same task) for advice.

An example of a (rather annoying) interface agent is the paperclip or office assistant found in Microsoft Office programs.

#### Heterogeneous (or hybrid) agents

Combining at least two different agent types leads to a heterogeneous or hybrid agent system. Heterogeneous agents need a common agent communication language and protocol to communicate and cooperate. More of this will be discussed in the next chapter.

The wide range of agents that have been developed makes it very difficult to categorize all agents and as with the definition of an agent there is no classification that researchers have agreed upon. One approach is to look at three dimensions or axes that can be used to measure an agent’s capabilities; agency, intelligence, and mobility as is demonstrated in Figure 3 [Gilbert et al 1995].

Figure 3: Gilbert’s scope of intelligent agents defined by

agency, intelligence and mobility

Agency refers to the degree of autonomy the agent has in representing the user to other agents, applications and computer systems. Intelligence deals with the ability of the agent to use domain-specific knowledge and its ability to solve the problem at hand. An agent is mobile if it can move between systems in a network. Of course, more dimensions can be added but three dimensions are easier to visualize.

Another classification often found in agent literature is the one devised by Nwana [1996] that uses several dimensions to classify software agents:

- Ideal and primary attributes: agents should exhibit several ideal and primary attributes like autonomy, learning and cooperation.

- Mobility: agents are classified as either static or mobile.

- Reasoning model: either deliberative or reactive.

- Role: e.g. information searching, management, process control or Internet.

- Hybrid: combination of two or more of the above.

- Secondary attributes: e.g. versatility, benevolence, veracity, trustworthiness, temporal continuity, ability to fail gracefully, and mentalistic and emotional qualities.

Figure 4: Part view of Nwana’s agent typology based on ideal and primary attributes

Note that the distinctions between the agents in Figure 4 are not strict. For example, with collaborative agents there is more emphasis on cooperation and autonomy than on learning but we do not imply that collaborative agents never learn. Likewise, for interface agents, there is more emphasis on autonomy and learning than on cooperation. Ideally, agents should do all three equally well, but this is the objective rather than the reality.

## Agent applications

Most applications of agents fall in one of two categories: simplifying distributed computing or acting as intelligent resource managers. The first is often used when the problem that needs to be solved is distributed. In this case agents are able to solve local problems and communicate to other agents in other locations or move through a network. In the second case, agents are used to automate certain processes and/or overcome user interface problems. Current direct-manipulation user interfaces are rather static. Agents allow more flexibility by running search or filtering operations in the background, delegating tasks to other agents, ‘waking up’ at certain scheduled times, adapting to new situations etc. Examples of application domains that use agents are:

- Industrial applications

- Process control (e.g. ARCHON agent, see also section 4.5)

- Manufacturing (robots)

- (Air) Traffic control

- Commercial applications

- Information management (data mining, filtering or gathering information on the WWW)

- Electronic commerce (automated trading)

- Business process management (workflow management)

- Medical applications (patient monitoring or information exchange between doctors)

- Learning (helping users to learn new computer programs)

- Entertainment (playing with or against virtual characters in computer games)

The range of firms and universities actively working on agent technology is quite broad and the list is growing. It includes small companies (e.g. Tryllian and Verity), medium-size organizations (e.g. General Magic, MIT, Delft University of Technology) and the real big multinationals (e.g. IBM, Lotus, Microsoft).

## Advantages and disadvantages

It is clear that agents are ideal for helping people to perform a task or by taking over tasks completely. Other advantages of agents are:

- Increased flexibility and adaptability: agents are able to learn and adapt to particular situations. Everyone can have their own personal agent that helps them as they see fit. The agents will adapt to the user’s preference.

- Abstraction and modularity: agents form a good way of abstracting functionality. They correspond to the general idea that many people have of a smart autonomous computer program. Also, parts of an agent can be replaced with new functionality without changing the interface to humans.

- Robustness: a well-designed agent with a good reasoning system can stabilize or resolve the situation in case of errors or misunderstanding.

- The possibility for parallelism: apart from agents working in parallel with humans, taking over their work, agents can work together to speed up the execution of the given task.

Someone once said that “every advantage also has its disadvantages” and of course this is also true for intelligent agents. Although agent technology has an important role to play in the development of leading-edge computing applications, it should not be oversold. Most applications that currently use agents could be built using non-agent techniques. A number of drawbacks common to all agent-based applications are:

- No overall system controller or global perspective: the actions chosen by an agent are determined by that agent’s local state. In many realistic agent systems complete global knowledge is not possible. This means that agents make sub-optimal decisions. An agent-based solution may not be appropriate for domains in which global constraints have to be maintained.

- Time constraint: the more sophisticated an agent is and the more functionality it has, the longer it takes for the agent to reach a decision. This makes it difficult for complex agents to function in environments where real-time response must be guaranteed. Also, learning agents might need a substantial amount of time to learn the appropriate behavior, if they are able to learn their task at all.

- User’s knowledge: people need to know how they can use agents to do certain tasks for them. This means that they need to know what the agent can and cannot do. The functionality of the agent should be clear but at the same time the agent’s complexity should be hidden from the user in order not to confuse him or her too much.

- Trust and delegation: for individuals to be comfortable with the idea of delegating tasks to agents, they must first trust them. Both individuals and organizations need to become more accustomed and confident with the notion of autonomous software components, if they are to become widely used. Users have to gain confidence in the agents that work on their behalf.

- Legislation and liability: should a user be held (partly) responsible for his or her agent’s actions? Who is liable for malfunctioning agents? These are matters that are not resolved yet.

# Multi-agent systems

Multi-agent systems are formed when multiple agents are combined to perform a certain task. Section 3.1 shows why this approach can be helpful. The next section (3.2) describes how agents can work together to solve a common task. Usually, this requires some kind of inter-agent communication and this is the subject of section 3.3.

## Introduction

Multi-Agent Systems (MASs) are systems in which two or more agents interact with each other to accomplish a task, usually through cooperation and coordination of their actions. The MASs approach is part of the field of Distributed Artificial Intelligence, but is largely influenced by other fields like robotics, psychology, ethology, biology and artificial life. Research on MASs focuses on mechanisms of self-organisation and the creation of agents that can accomplish complex tasks by working together.

### Why use multi-agent systems?

The concept of intelligent agents is not as revolutionary as it might seem at first glance. All the used techniques, like information filtering, planning, autonomous behavior and adaptive control have already been used successfully in various applications. The difference with agents is that they combine all these techniques and form a new and more intuitive way of creating smart applications. The idea is that one and one is not two, but could be three or even more. This is called synergy; the sum of the parts is larger than the whole. As Marvin Minsky described in his book “The society of the Mind” [Minsky 1986] intelligence is not found in some central processor, but in the collective behavior of a large group of specialized and interconnected entities. Even with a group of very simple agents, highly complex intelligence can emerge as the result of interaction between agents, coordination, and careful selected behaviors.

There are many possible applications of MASs but the most important ones fall into one of four categories:

- Program design: MASs can be used in computer programs, which are capable of interaction, adaptation, and possibly even reproduction, in order to provide more flexibility to their users. An example is cooperating interface agents, negotiating with other agents on behalf of the user.

- Simulation: agents can be used to directly model the actions and behaviors of individuals in a simulated environment. These simulations are often called microscopic simulations, micro-simulations, or micro-worlds to denote that the used representations are at the level of the individual.

- Collective robotics: a group of robots can cooperate in accomplishing their task and individual robots need to coordinate their actions in order not to bump into each other

- Distributed problem solving: this involves all situations in which agents are used to solve a problem other than those mentioned above. Both the problem and the expertise of the agents can be distributed. Homogeneous agents can be applied to solve a distributed problem (e.g. sending e-mail) or specialized heterogeneous agents can solve a central problem (e.g. robots creating an industrial product).

## Interaction, cooperation and coordination

Interaction between agents occurs when two or more agents form a dynamic relationship through influential actions [Ferber 1999]. Agents can interact through a series of events, during which they are in contact with each other in some way. This contact can be direct, via another agent or indirect through a mutual environment. Direct interaction and interaction via other agents usually occurs by sending messages. When interaction takes place through changes in a mutual environment no communication is necessary.

Interaction situations can be classified according to three criteria; the objectives or intentions of the agents (compatible or incompatible goals), the relationships of these agents to the available resources (e.g. space, time, energy) and the skills available to the agents to achieve their goals. The eight possible situations are shown in Table 1 below [Ferber 1999].

Table 1: Types of interaction

| Goals | Resources | Skills | Types of possible situations |
| --- | --- | --- | --- |
| Compatible | Sufficient | Sufficient | Independence |
| Compatible | Sufficient | Insufficient | Simple collaboration (e.g. communication) |
| Compatible | Insufficient | Sufficient | Obstruction (e.g. traffic jams) |
| Compatible | Insufficient | Insufficient | Coordinated collaboration (e.g. industrial activities) |
| Incompatible | Sufficient | Sufficient | Individual competition (racing) |
| Incompatible | Sufficient | Insufficient | Collective competition (grouping, team competitions) |
| Incompatible | Insufficient | Sufficient | Individual conflicts over resources |
| Incompatible | Insufficient | Insufficient | Collective conflicts over resources (companies, war) |

If we look at Table 1, the most promising and useful situation would be a MAS in which the agents are able to cooperate. Cooperation can improve agent performance, resolve conflicts and increase ‘survival capacity’.

Once a group of agents decide to cooperate, coordination problems may arise. The question is which agent has to do what, using what resources? Agents may need information and results, which only other agents can supply, and resources can be limited. Coordination is a form of cooperative behavior and consists of all the extra actions that need to be done in order to manage a group of agents. It is not directly productive but serves to improve the way in the group of agents function. The easiest way to resolve coordination problems is to break down the task at hand into subtasks and distribute the subtasks over the agents. This task allocation technique can be performed in several ways, with the three most common being:

- Centralized allocation: specifically designed for coordination with one central ‘leader’. This is fairly easy to implement, but the disadvantage is that cooperating agents have to know each other. Centralized allocation can be separated into two sorts:

- Imposed allocation via hierarchical structure: one agent gives orders to its subordinates.

- Allocation by trader or broker: a special agent knows the capabilities of all other agents and mediates between them.

- Distributed allocation: in this case, given an open set of agents, the possibility to coordinate and cooperate exists but it is not obligatory. Each agent individually tries to obtain the services it needs from other agents. Distributed allocation can also be separated into two types:

- Allocation by acquaintances: agents know all other agents and their capabilities, and send requests to other agents to perform the task.

- Allocation by contract net: bidding requests are sent to all agents and the agent that makes the best offer is allowed to perform the task.

- Emergent: emergent cooperation is characteristic for reactive systems. Each agent is designed to perform a particular task and usually no negotiation during the job is necessary. An example is reactive agents that use potential fields which are imaginary attracting or repelling fields, to avoid other agents [Arkin 1998]. The strength of this approach is the speed and simplicity of the agents, the disadvantage is the limited and low-level intelligence of the agents.

Not all tasks or subtasks can be done by one agent alone, so task allocation is not always sufficient. Some tasks require more sophisticated methods of coordination, such as planning, synchronization (sequencing of actions) and pre-determined regulations (set of rules of behavior for all agents).

## Communication

One of the most important aspects of multi-agent systems is communication. If agents do not communicate they cannot tell other agents about their intentions. A good communication system allows agents to communicate effectively and exchange information despite possible differences in hardware platforms, operating systems, programming languages, and reasoning systems. Communication can be either synchronous or asynchronous. In the first case the agent sends a message and waits until it receives a reply. With the latter the agent sends a message and continues his work without waiting for an answer.

Communication can be implemented either as message passing or using shared variables. Message passing is considered a better approach because it is simpler to implement and no synchronization methods such as semaphores are needed. Note that there is a trade-off between computation (agents maintaining models of the situation) and communication costs. The more an agent stores in its memory, the more local computation is required to maintain a world model and reason about it, but the lower the communication overheads.

Many different agent communication protocols exist, but the most important ones are KQML, FIPA’s ACL and KIF. The relationship between them is shown in Figure 5.

Figure 5: Composition of an agent message

#### Knowledge Query and Manipulation Language (KQML)

KQML, which stands for Knowledge Query and Manipulation Language, is a language and protocol for exchanging information and knowledge between software agents. KQML has been developed as part of the DARPA Knowledge Sharing Effort, a consortium that is “aimed at developing techniques and methodology for building large-scale knowledge bases which are sharable and reusable” [Finin et al. 1994].

KQML defines both the message format and the message-handling protocol to support run-time knowledge sharing. It is not an interpreted or compiled language but a set of rules that can be used in communicating. KQML messages, which are called performatives, express attitudes regarding the content of the information exchange and allow agents to find other agents that are able to process their requests. Performatives are combined to form KQML exchanges (conversations). The content field of each performative is not dictated by KQML and can contain any kind of data. All KQML implementations ignore the content field of the message, except that they need to recognize where it begins and where it ends.

Although there is a predefined set of KQML performatives, agents may choose to process only a few performatives or use other performatives of their own. Reserved performatives must be implemented in the standard way. Specifications for the full set of reserved KQML performatives can be found in [Labrou and Finin 1997a]. The following is an example of a KQML message:

(ask-if	:sender		A

:receiver		B

:language		prolog

:ontology		workload_assessment

:reply-with	id1

:content		“pilotstatus(stress)” )

The first word of the KQML message is the name of the performative. In this ‘ask-if’ performative agent A is querying agent B in the Prolog language about the truth value of ‘pilotstatus(stress)’. It is up to agent B to interpret the content of the message and evaluate the query. The word ‘:ontology’ is used to provide additional information about the domain related to the content of the message. Any responses to this KQML message will be identified with ‘id1’.

Early experiments with KQML started in 1990 and current KQML implementations use standard communication and messaging protocols as a transport layer, for example TCP/IP, SMTP, HTTP and CORBA. Although there have been attempts to draw up the semantics of KQML [Labrou and Finin 1997b], they are not well-defined and it is quite common that different people use the same performative with a different meaning [Skarmeas 1999].

#### FIPA’s Agent Communication Language (ACL)

The FIPA Agent Communication Language (ACL) is the European counterpart of KQML. The Foundation for Intelligent Physical Agents (FIPA) is an international non-profit association that “develops specifications of generic agent technologies to provide a high level of interoperability across applications”. The target of FIPA is to create ‘Intelligent Physical Agents’, devices intended for the mass market, capable of executing actions to accomplish goals set by or in collaboration with human beings or other intelligent agents with a high degree of intelligence [FIPA 1997].

The FIPA organization has produced a specification of an agent communication language that looks similar to KQML but with much better defined semantics. The exact specifications can be found on the FIPA homepage on the Internet [FIPA 2001].

#### Knowledge Interchange Format (KIF)

The Knowledge Interchange Format (KIF) is a formal language for the interchange of knowledge between different computer programs. Based on first order logic, KIF was specifically designed to act as a mediator or interlingua in the translation of other languages. Translation occurs by transforming statements from language A to KIF and from KIF to language B. KIF can also be used as a common language between two agents that use different native languages or to describe programs or scripts for agents to follow. It is not meant to be a programming language or internal knowledge representation of an agent, but can be used as such. Given the prefix syntax of KIF, it looks similar to LISP, Scheme or CLIPS. An example of a KIF sentence is:

(defrelation bachelor (?x) :=

(and (man ?x) (not (married ?x))))

The sentence above says that variable x is a bachelor, if x is a man and x is not married.

As with KQML, KIF is part of the DARPA Knowledge Sharing Effort, but unlike KQML, the KIF language description includes a specification for both the syntax and the semantics. The complete specification of KIF can be found in [Gensereth and Fikes 1992].

# Building agents

The first section of this chapter briefly outlines the general agent construction process. Section 4.2 and 4.3 cover two important aspects of agents, namely the knowledge representation language and the knowledge base. Section 4.4 shortly addresses agent programming. Finally, the last section describes several existing agent architectures that can be used as a guideline or template for constructing agents.

## The construction process

Much literature has been written about the construction of agents, but little has been said about the general design process. Most papers and books deal with the construction of specific agents or agent types. Naturally, the followed course of design depends very much on the type of agent and its application, but in any case some guidelines can be given. Generally speaking, every agent can be constructed by going through the following phases:

- Analyzing the problem domain: identify the significant objects and features of the application domain. What are the percepts that the agent can encounter and what kind of actions can be done to influence the environment?

- Defining the agent’s requirements: what goals or performance measures is the agent supposed to achieve? How should it respond in certain situations?

- Choice of knowledge representation language and reasoning model: based on the domain knowledge that needs to be modeled a choice is made for a knowledge representation language and reasoning model.

- Building the (initial) knowledge base: by storing facts and rules into a knowledge base the agent’s behavior and inner-workings are defined.

- Testing the agent: just like in any other design process, the agent needs to be tested to verify that the agent works correctly and its requirements are met.

Other design or construction phases might be necessary depending on the type of agent or application, for instance creating a learning mechanism, designing a graphical user interface, or adding specific communication protocols. With some agents it might be necessary to ‘train’ them first in order to fill their knowledge base with useful information.

## Knowledge representation and reasoning

Humans store knowledge in the various parts of their brain (scientists are still trying to figure out how it works exactly), but since computers or agents do not have a brain, they must use an alternative way. With agents, a computer memory is filled with certain symbols, numbers and characters that represent a certain fact or belief. Each symbol represents an object or idea and this is called ‘knowledge representation’. A knowledge representation language defines the syntax and semantics for expressing knowledge. It should be capable of expressing all the necessary descriptions in an appropriate and effective way. What our agent is expected to do and in what domain will have a significant impact on the type of knowledge representation. Several important features should be considered [Tecuci 1998]:

- Representational adequacy: the ability to represent all of the kinds of knowledge that are needed in a certain application.

- Inferential adequacy: the ability to use the available knowledge to deduce new information (reasoning).

- Problem-solving efficiency: the ability to represent and create the knowledge and procedures that are needed to solve the problem.

- Learning efficiency: the ability to easily acquire and learn new information and integrate it within existing knowledge structures.

Very much related to the choice of the knowledge representation language is the reasoning system, also called inference system. An agent’s reasoning system consists of data structures for storing knowledge and, more importantly, procedures for manipulating these structures and deducing new knowledge. The choice of knowledge representation dictates the type of reasoning.

The communication protocols mentioned earlier in section 3.3 are forms of knowledge representation languages. Other examples of often-used knowledge representations and reasoning systems are production systems, fuzzy systems Bayesian networks, semantic networks and frames. Many other (but less used) knowledge representations exist and the interested reader is referred to [Russell and Norvig 1995] for more detailed information.

#### Production systems

Production systems can very well represent what to do in certain predetermined situations, but are less adequate for representing knowledge about objects. The IF-THEN rules used by production systems are very easy to understand and new knowledge can be added easily. This is the reason that production systems have become one of the most popular forms of declarative knowledge representations in artificial intelligence. A typical production system consists of the following parts:

- Knowledge base: permanent memory containing IF-THEN rules.

- Working memory: temporary memory containing new or derived facts.

- Inference engine or matching mechanism: reasoning logic used to produce new data.

- Conflict resolution mechanism: procedure that decides which rules should be executed.

A production system works by matching the facts stored in the working memory to the IF-part of the rules stored in the knowledge base. The conflict resolution mechanism chooses the best rule of all matching rules and then the THEN-part of the best rule is executed.

An example of a production system is the ‘C Language Integrated Production System’ or CLIPS for short [CLIPS 2001].

#### Fuzzy systems

Fuzzy control systems produce actions using a set of fuzzy rules based on fuzzy logic. In conventional logic assertions about the world are either true or false, there is nothing in between. Values such as true and false (1 and 0) are referred to as crisp, that is, they have one exact meaning. Fuzzy logic allows variables to take on other values (between 0 and 1), determined by how much they belong to a particular set. In fuzzy logic these variables are referred to as linguistic variables, which have non-crisp meanings (e.g. fast, slow, far, big, etc.). Membership functions measure the degree of similarity an instance of a variable has in its associated fuzzy set. A fuzzy logic control system consists of the following:

- Fuzzifier: maps a set of crisp inputs to a collection of fuzzy input sets.

- Fuzzy rule base: contains a collection of IF-THEN rules.

- Fuzzy inference engine: maps fuzzy sets onto other fuzzy sets according to the rule base and membership functions.

- Defuzzifier: maps fuzzy output sets onto a set of crisp output commands.

The main advantage of fuzzy systems is that they are more flexible than conventional rule-based systems and are often better understandable by humans due to the use of linguistic variables.

#### Bayesian networks

Bayes’ theorem says that the conditional probability that event Y will occur given that event X already occurred can be computed, providing we know the prior probabilities that X and Y could happen, and the conditional probability that X will occur when we know that Y has already occurred. In mathematics this is described as:

|  | P(Y ¦ X) = P(X ¦ Y) * P(Y) / P(X) | Equation 4-1 |
| --- | --- | --- |

A Bayesian network, also called a belief network, causal network or probabilistic network, is a data structure that uses this theorem and represents dependence between variables. Each variable has a corresponding node with a conditional probability table defining the relationships between its parent nodes. This makes Bayesian networks ideal for reasoning with uncertainty.

#### Semantic networks

Semantic networks are graphs in which the nodes represent objects, situations or events, and arcs between the nodes represent the relation between them. Semantic networks are well suited to represent knowledge about objects, but cannot represent processes very well. Their inferential capacity is fairly high, but their learning capacity is rather low due to the difficulty of adding new nodes to existing networks.

#### Frames

A frame is a collection of attributes that define the state of an object and its relationship to other frames (objects). A frame contains slots that represent attributes and fillers or scripts, which are attached procedures that are called when the value of a slot changes. Frames are often linked into a hierarchy to represent the hierarchical structure of objects. In this case, frames at a certain level can inherit knowledge from higher level frames.

Frames and semantic nets are closely related, both represent objects and their relations with other objects, and both have the same advantages and disadvantages. The main difference between the two representations is their syntax.

## The knowledge base

The most difficult part of developing an intelligent agent is the encoding of knowledge in the agent’s knowledge base and the modification of this knowledge through learning or reasoning. A knowledge base is a set of representations of facts about the world. Each individual representation or fact, also called sentence, is expressed in the used knowledge representation language. Certain conclusion might be drawn from the stored knowledge by using inference. For example, if it is known that “IF a THEN b” and fact ‘a’ is known to be true, then fact ‘b’ must also be true and can be stored separately in the knowledge base. The rules for drawing these conclusions are also part of the knowledge base.

The process of building a knowledge base is called knowledge engineering. A good knowledge base is clear and correct. The relations and facts that matter should be defined and the irrelevant details should be ignored. The construction of an intelligent agent’s knowledge base can be divided into three phases [Tecuci 1998]:

- Systematic elicitation of expert knowledge: the basic terminology (ontology) is developed and the conceptual structure of the knowledge base and knowledge representation language are chosen.

- Knowledge base refinement: the knowledge base is extended and debugged. After this phase the knowledge base should be complete and correct enough to provide solutions for most problems.

- Knowledge base reformulation: the knowledge base is reorganized to improve its efficiency.

A knowledge base can be created after a choice has been made which knowledge representation to use. In the early days of expert systems the knowledge engineer, someone who knows how to build a knowledge base, worked together with the domain expert and transformed the knowledge of the domain expert into a form usable by the reasoning system. The disadvantage of this approach is that the domain experts cannot always formulate their knowledge exactly. In addition, many domain experts are reluctant to work with a knowledge engineer in fear of their job being taken over by the developed expert system.

A different approach can be taken with machine-learning techniques. Using historical data from databases or examples generated by experts, the agent is trained to perform classification and prediction tasks without going through the expensive knowledge acquisition process. Verification of the learned knowledge can be done through experimental testing on data sets different from those on which learning was performed.

Building and maintaining the knowledge base is easier if the agent can learn, but building a good learning engine is not an easy task. It is very difficult to start with a blank knowledge base and fill it with the appropriate knowledge using machine-learning techniques (tabula rasa learning). A combination of hard encoded knowledge and machine-learning techniques seems to be more successful.

## Programming languages

Eventually, all agents have to be programmed using some sort of programming language. The most used agent-programming languages today are object-oriented languages. The main advantages of object-oriented languages over more traditional imperative languages is the reusability mechanisms they provide mainly through the use of classes, inheritance of object properties and methods, and the modular style of programming and design.

There are many different sorts of programming languages and language extensions that support the creation of agent applications. However, most of these language extensions focus on the creation of mobile agents on the Internet

One of the most used languages for agent programming is Java [Bigus and Bigus 1998]. Java is designed as a simpler version of C++ with no pointers. It supports multi-threading, single inheritance and a very limited form of multiple inheritance via so called ‘interfaces’.

An advantage of Java is that it is portable and ‘architecture neutral’. This means that Java code is compiled into ‘bytecodes’ that can be sent over a network and executed on any target machine as long as it uses the Java Virtual Machine. An example of a Java extension that can be used for agent programming is JESS, the Java Expert System Shell, a production system in Java.

Another language that is often used for agent programming is C++. This is also an object-oriented language and its main strong points are that it fast, powerful and widely used and supported.

## Architectures

An agent architecture can be defined as a particular methodology for building agents. It specifies how the various parts of an agent can be assembled in such a way that it can accomplish the necessary actions and how these parts should interact with each other.

#### Blackboard-based architectures

Originally designed for the HEARSAY-II speech-understanding system [Hayes-Roth 1985], the blackboard architecture is one of the most widely used architectures in symbolic multi-agent systems. The blackboard architecture is not very specific and can be seen as a ‘meta-architecture’, an architecture for implementing other architectures.

The blackboard model is based on a division into independent modules that do not communicate any data directly, but interact by sharing data in a common storage as is shown in Figure 6.

Figure 6: The blackboard model

A number of asynchronous information sources communicate with each other by placing and reading information from a shared resource called the blackboard. Only one resource at a time has access to the blackboard and a special control unit regulates the access. When another information unit wants access to the blackboard it sends a request to the control unit that schedules the request.

Blackboards form a good way of communication between heterogeneous information sources. The sources do not need to know about each other, the only thing they see is the blackboard.

#### The subsumption architecture

In the mid-1980s, Rodney Brooks developed the subsumption architecture [Brooks 1986]. Brooks argued that the traditional ‘sense-plan-act paradigm’ used by the artificial intelligence community so far, was not good enough. He believed that complex behavior does not necessarily need a complex control system but can be the result of multiple simple rules working on the same task, a rather new idea at the time.

The subsumption architecture is a layered architecture that uses arbitration strategies and augmented finite state machines as its basis. Behaviors in the subsumption architecture are represented as separate layers with individual layers working in parallel on individual goals. The lower layers have no awareness of higher layers. Higher layers can be added on an already working control system without further modifications to the lower layers. The first behavior of an agent might simply be to avoid objects (see also Figure 7). Another higher-level behavior might be to move in a given direction. This behavior will dominate the obstacle-avoidance behavior by suppressing its output to the actuators unless an object is too close. The higher levels subsume the lower levels, hence the name of the architecture.

Figure 7: Subsumption behavior decomposition of an autonomous robot

The subsumption architecture has been the foundation for reflex agents and the behavior-based robotics approach [Arkin 1998], [Ehlert 1999]. The largest influence of the subsumption architecture on both is the direct coupling of perception and action using simple behavioral schemes.

#### ARCHON

ARCHON is part of the European ESPRIT program and stands for ARchitecture for Cooperative Heterogeneous ON-line systems. ARCHON is the name of the consortium of 14 organizations as well as the name of the architecture they devised. The general goal of ARCHON is to create a heterogeneous multi-agent system for industrial applications, but it has focused on a specific application, which is electricity management. Each ARCHON agent supervises and controls a part of the total system and cooperates with other ARCHON agents. The ARCHON agent consists of five components [Wittig 1992]:

- High-level communication module: handles the communication with other agents.

- Acquaintance model: contains information about other agents (e.g. skills, interests, status).

- Planning and coordination: initiates cooperation requests and deals with requests from other agents.

- Self-model: contains the agent’s own state, an abstract model of the system that the agent controls, and plans on how to control it.

- Monitor: provides the interface between the other components and the controlled system, checks the state, and starts or stops certain tasks within the system.

The relation between the five components is shown in Figure 8.

Figure 8: An ARCHON agent

#### The Belief-Desire-Intention architecture

Developed by Anand Rao and Michael Georgeff, Belief-Desire-Intention agents (BDI-agents) are characterized by a mental state with three components [Rao and Georgeff 1995]. The beliefs of an agent are its model of the world and its desires contain the goals it wants to reach. The intentions are the actions that the agent has decided to do in order to fulfill its desires. If an agent receives sensor input, it will update its world model with the use of a belief revision function. The agent then generates options for actions (desires) with an option generation function. A filter function updates the agent’s intentions according to its current beliefs, desires and past intentions. At the end of the cycle the action selection function chooses the action that is expected to perform the best according to the agent’s current intentions.

# Agents in the ACE System

In the first section a short description of the ACE system is given. The following sections describe several parts of the ACE system in more detail. Section 5.2 deals with the input of the system and how it is processed and filtered by the workload assessment and situation assessment modules. The next section, which is 5.3, describes the used knowledge in the reference module. The agents or logical unit that form the heart of the system are discussed in section 5.4. Finally, in section 5.5, possible output methods are mentioned.

## Description of the ACE system

Basically, there are two things that need to be determined by the ACE system. First of all, the system has to determine the pilot’s information needs (content) and secondly the system must decide on a suitable format to present this information (modality).

In the figure below a proposal schema for the ACE system is shown. The schema is a slightly different version of the ACE system that was described in the ACE technical proposal [NLR 2000]. All the components described in the technical proposal are still present, only the positions of the workload assessment and situation assessment (SA) modules have changed. The idea is that both modules should provide high-level data for the agents that determine the actual output of the system.

Figure 9: The ACE system

As you can see in Figure 9, input from different types of sensors is received by an input-processing component. The job of this component is to support the rest of the system (agents) by extracting important measurements regarding the mission progress and the workload and situation awareness of the pilot. Four different agents are responsible for adapting the interface or automating certain tasks, with each agent responsible for a specific aspect of the mission. The agents and the input-processing component use (static) knowledge that is available in the reference module. Mission or pilot-dependant information can be entered into this module before the flight.

## Sensors and input processing

The information the pilot is interested in will be dependant on his current situation, so the Situation Assessment and Recognizer (SAR) will play an important role in determining what information is wanted by the pilot. The format and modality that will be used to display the information will depend on the pilot’s state and this will be determined by the Workload and situation Awareness Module (WAM). Both the SAR and the WAM look at the received input based on some current situation (if available). This means that they expect to receive some type of information depending on the information received earlier. For example, if the SAR has determined that the pilot has just taken off, chances are that the situation will develop to a situation of normal (in-flight) operation and not for example landing. Knowledge of situation development or pilot state can simplify or speed up input processing.

Pre-processing of sensor information should be done either at the sensors or as the first step in the input processing module.

### Situation Assessment and Recognizer (SAR)

Pilots need different information in different types of situations. Below we list several possible situations or stages during a mission:

- Take-off

- In-flight, normal operation

- Approaching mission target

- Under attack (incoming missile)

- Dog-fight

- Landing

If we look at specific situations or events during a mission, we can discriminate between two different types. The first type has to do with the plane itself and the status of its components, the second deals with events outside the plane. Below we give some examples of both types:

Airplane

- Engine status (failure or running ok)

- Fuel remaining

- Available weapons (amount of missiles/bombs left)

- Radar status (operational or failure, current range)

- Auto-pilot status (on or off)

Environment

- Weather conditions (cloud level, wind)

- Presence and location of enemy aircraft

- Presence and location of enemy radar / SAM installations

- Location of the plane

- Location of targets

- Incoming enemy missile

### Workload and situation Awareness Module (WAM) 2

Special measurements to determine the mental workload and situation awareness of the pilot are needed. Possible methods to determine this can be:

- Heart rate or pulse; a rising heart rate means more anxiety and more stress.

- Voice analysis; stressed pilots talk faster and louder than normal. Also, pilots can give commands to the adaptive interface using speech recognition technology.

- Gaze tracker; the direction the pilot is looking, his pupil size, and eye blink rate can be determined with the use of a gaze tracker. The direction the pilot is looking gives an indication of his situation awareness. Pupil size and eye blink rate are indicators of his mental workload.

- EEG measurements; indicator of mental workload.

Besides these methods, information from the SAR can be used to help determine the possible workload and situation awareness of the pilot. For example, if the pilot is approaching his mission target, his mental workload will most probably rise.

Research should be done to assess if the techniques mentioned above are adequate and reliable enough to measure different degrees of workload and SA.

### Uncertainty of measurements

Both the SAR en the WAM will most likely not be capable to give an exact measurements in all situations. For example, if two separate measurement techniques contradict each other the system will have difficulty in deciding which one is false. Therefore, these systems need to be able to deal with uncertainty. Possible solutions might be the use of belief networks or fuzzy rules.

## Reference component

The reference module contains several databases or rule sets that deal with distinct topics. It is possible to create one large database or rule set, but creating multiple smaller databases allows for easy reference and simpler database construction.

The databases should not only contain information about mission plans and expected situations, but also how to deal with unexpected events such as engine failure.

## Agents

Four important aspects regarding a mission are distinguished; the goals of the mission itself, the pilot’s status, the plane status and the environment. We propose to construct four separate agents, one for each of these aspects. Each agent should primarily be concerned with its own topic and goals, but must also be able to cooperate and negotiate with the other agents.

#### The mission agent

The mission agent looks primarily at the output of the SAR module and the mission plan in the reference module. Important issues for the mission agent are the mission progress and the location and distance of targets.

#### The environment agent

The environment agent also looks at the output of the SAR module and compares it to the information that is known about the environment that is stored in the reference module. The main task of the environment agent is to alert the pilot for unexpected objects or events such as ground threats.

#### The plane agent

The plane agent monitors the status of the airplane and reports any malfunctions. The plane agent uses knowledge that is stored in the ‘plane’ database of the reference module.

#### The pilot agent

The pilot agent is the most important agent of all. It looks at the available pilot model and pilot preferences and has knowledge of the interface and equipment that can be used to alert the pilot. All agents need to communicate to the pilot agent the information they want to provide to the pilot. The pilot agent decides what modality to use and whether or not to filter or block this information. This decision will depend on the mental workload and situation awareness of the pilot.

The design of each agent will depend largely on the (form of) information from the reference module and the SAR and WAM. Therefore, we should investigate these components further before constructing the agent models. However, in Figure 10 we show a general model that can be used.

Figure 10: possible general agent model

All input that is received by an agent is stored (temporarily) in its memory for reference. The input is also send to the rules or behaviors of the agent. A subsumption architecture or similar structure can be used to model these behaviors. Low-level and simple behaviors, for example ‘warn for enemy planes or missiles’ generate output that is immediately determined and sent forward. Higher-level and more complex behaviors need more time and information to determine their output. An example of a higher-level behavior can be ‘Show mission schedule’.

## Output

A multimodal pilot-plane interface should take advantage of a human’s capacity for parallel processing through different sensor modalities such as vision, sound and feeling. This means that output can be given in three different ways:

- Display visual information (on HUD or other available displays).

- Generate sound (e.g. spoken warnings or beeps to attract attention).

- Force feedback (e.g. vibration through stick or chair).

Research needs to be done to investigate which modality is best given a certain situation and to determine how pilots react to the different modalities. Especially with force feedback, we need to determine if feedback through vibration is a feasible and desirable method to implement. Pilots may be uncomfortable with vibrating objects or may not be able to determine the difference between the ‘normal’ vibration of the plane during flight and the vibration generated by the system.

Another consideration that needs more research is the use of 3D audio to give the pilot a sense of direction of where the sound is coming from. This might be useful to quickly indicate the location of missiles or enemy aircraft to the pilot.

Of course, individual preferences of pilots need also be taken into account.

# Bibliography

Arkin, R. C. (1998) “Behavior-based robotics”, The MIT Press, Cambridge, Massachusetts.

Bigus, J.P. and Bigus, J. (1998) “Constructing intelligent agents with Java: a programmer’s guide to smarter applications”, Wiley & Sons, Inc.

Bradshaw (1997) “Software agents”, AAAI Press / The MIT Press, Cambridge, Massachusetts.

Brooks, R.A. (1986) “A robust layered control system for a mobile robot”, MIT AI Lab Memo 864, September 1985. http://www.ai.mit.edu/people/brooks/papers/AIM-864.ps.Z (sep 27, 2001)

Also in IEEE Journal of Robotics and Automation Vol. 2, No. 1, March 1986, pages 14-23.

CLIPS (2001) http://www.ghg.net/clips/CLIPS.html (sep 27, 2001)

Ehlert, P.A.M. (1999) “The use of artificial intelligence in autonomous mobile robots”, research report, Delft University of Technology. ftp://ftp.kbs.twi.tudelft.nl/pub/kbs/thesis/MSc_Thesis_Ehlert,Patrick/research_report.pdf (oct 1, 2001)

Endsley, M.R. (1999a) “Situation awareness in aviation systems”, in Handbook of Aviation Human Factors, Garland, D.J., Wise J.A., Hopkin, V.D. editors, Lawrence Erlbaum Associates, Mahwah, NJ USA.

http://www.satechnologies.com/Papers/pdf/AviationSA-chp.pdf (oct 1, 2001)

Endsley, M.R. (1999b) “Situation awareness and human error: designing to suppport human performance”, in Proceedings of the High Consequence Systems Surety Conference, Alburquerque, NM USA.

http://www.satechnologies.com/Papers/pdf/Sandia99-safety.pdf (oct 1, 2001)

Ferber, J. (1999) “Multi-agent systems: an introduction to distributed artificial intelligence”, Addison Wesley Longman Inc., New York.

Finin, T., Labrou, Y. and Mayfield, J. (1994) “KQML as an agent communication language”, in Proceedings of the Third International Conference on Information and Knowledge Management (CIKM'94), ACM Press, November 1994.

Also http://www.cs.umbc.edu/agents/introduction/kqmlacl.ps (sep 27, 2001)

FIPA (1997) “Agent communication language”, technical report of the Foundation for Intelligent Physical Agents. http://www.cselt.it/fipa/spec/fipa97/f8a22.zip  (sep 27, 2001)

FIPA (2001) http://www.cselt.it/fipa or http://www.fipa.org  (sep 27, 2001)

Gensereth, M. and Fikes, R. (1992) “Knowledge Interchange Format, Version 3.0. Reference Manual”, Logic Group Report Logic-92-1, Stanford University. http://logic.stanford.edu/papers/kif.ps  (sep 27, 2001)

Gilbert, D., Aparicio, M., Atkinson, B., Brady, S., Ciccarino, J., Grosof, B., O’Conner, P., Osisek, D., Pritko, S., Spagna, R. and Wilson, L. (1995) “IBM Intelligent Agent Strategy”, IBM corporation.

Hayes-Roth, B. (1985) “A blackboard architecture for control”, in Artificial Intelligence Vol. 26, pages 251-321.

Jennings, N. R. and Wooldridge, M.J. (1998) “Agent technology: foundations, applications, and markets”,  Springer-Verlag, Berlin, Germany.

Labrou, Y. and Finin, T. (1997a) “A proposal for a new KQML specification”, technical report CS-97-03, University of Maryland, February 1997. http://www.csee.umbc.edu/~jklabrou/publications/tr9703.ps  ((sep 27, 2001)

Labrou, Y. and Finin, T. (1997b) “Semantics and conversations for an agent communication language”, In Readings in Agents, Huhns, M. N and Singh, M.P. editors, Morgan Kaufmann Publishers, Inc.

Also in Proceedings of the Fifteenth International Joint Conference on Artificial Intelligence (IJCAI-97), August 1997. http://www.cs.umbc.edu/~finin/papers/ijcai97.ps  (sep 27, 2001)

Maes, P. (1994) “Agents that reduce work and information overload”, in Communications of the ACM, Vol. 37, No. 7, pages 31-40, ACM Press, July 1994.

Also http://pattie.www.media.mit.edu/people/pattie/CACM-94/CACM-94.p1.html (sep 27, 2001)

Minsky, M. (1986) “Society of the mind”, Simon & Schuster, New York.

NLR  (2000) “Adaptive Cockpit Environment”, memorandum VE-2000-002, Version 1.1, Nationaal Lucht-en Ruimtevaartlaboratorium, The Netherlands.

Nwana, H.S. (1996) “Software agents: an overview”, In Knowledge Engineering Review Journal, Vol. 11, No. 3, November 1996.

Also http://www.labs.bt.com/projects/agents/publish/papers/ao4.ps.gz (sep 27, 2001)

Rao, A. S., and Georgeff, M. P. (1995) “BDI Agents: From Theory to Practice”, in Proceedings of the 1st International Conference on Multi-Agent Systems (ICMAS-95), San Francisco, USA, June,  pages 312-319.

Russell, S. and Norvig, P. (1995) “Artificial Intelligence: a modern approach”, Prenctice-Hall International, Inc.

Shoham, Y. (1993) “Agent-oriented progamming”, in Artificial Intelligence 60, pages 51-92.

Shoham, Y. (1997) “An overview of agent-oriented programming”, in Software agents. Bradshaw, J.M editor, AAAI Press / The MIT Press, Cambridge, Massachusetts, 1997.

Skarmeas, N. (1999) “Agents as objects with knowledge base state”, Imperial College Press, London.

Tecuci, G. (1998) “Building intelligent agents: an apprenticeship multistrategy learning theory, methodology, tool and case studies”, Academic Press, San Diego, California.

Wittig, T. (1992) “ARCHON: an architecture for multi-agent systems”, Ellis Horwood Limited, England.

The terms ‘ARPA’ and ‘DARPA’ (Defense Advanced Research Projects Agency) are both used in literature and denote to the same organization. Also the terms ‘Knowledge Sharing Initiative’ and ‘Knowledge Sharing Effort’ refer to the same consortium.

This information is incomplete or possibly incorrect. More information about the SAR and WAM should be provided by NLR or Brno. Nevertheless, a short description of the SAR and WAM is provided in order to present some ideas at TUD about the two modules.