Take Off	 						1F-16C-1CL-1

1. Tower				             Wait/ Cross/Enter Runway

2. Master Arm			                          Auto

3. Radio					Chan Select

4. Controls 					Full + Free

5. DED					             F-Ack

6. Caution Panel				Chk.

7. SpeedBrakes					Chk disengage.

8. Line-up					Line Up

Action/Note:

Apply Power steadily, 150knts, 10 degrees positive pitch.

Warning:

Do not exceed 14 degrees pitch.

# Airbourne

12. Gear				             Raise Gear.

13. Airspeed					Keep IA <200

14. U/C					             Check Locked (3xG)

15. Radar				             Set  RAD. ACM.

16. DED/Caution Panel				F-ACK / Chk

17. MFD				             Cycle/ Req data

18. Maneuver 				             Turn to next stp.

# Aborted T/O

1. Speedbrake					Extended

2. Throttle					Idle

3. Pitch Angle				             7 degrees

4. Wheel Brakes		                         Full Engaged

5. Maneuver 				            Halt A/C

6. Maneuver 				            Clear Runway (not req.)

7. DED/CAUTION		                         Chk/Exit A/C if REQ.

NOTE:

If IAS <110 at ½ runway length, declare Emergency and EJECT.

USAF/DOD 11.2565.67			                                   F-16C-BL*50-52