Rejoin	 						1F-16C-1CL-1

1. Tower (26nm out)			             Call Inbound.

2. Master Arm			                          Auto

3. Radio					Chan Select

4. HUD					             De-Clutter

5. DED					             F-Ack

6. Caution Panel          			             Chk.

7. MFD						Chk / Cycle

8. Line-Up					Align with airfield

9. Respond					Respond to tower vectors

10. Circuit					Over-Fly runway

11. Circuit 					Join circuit ~220knts

12. Circuit					Turn into Crosswind

13. Crosswind Leg				Tower/ Permission to Land.

14. Crosswind leg 				Checks.

Note:

Unless previously cleared  aerobatic maneuver is not permitted over the airfield. Pitch and Bank should not exceed 70. IAS <400.

# Crosswind Checks

1. Gear				                          Up

2. Airspeed					Keep IA ~250Knts

3. Radar					No Radar

# Downwind Checks

1. Vis				                          Vis chk of A/C

2. DED/Caution Panel				F-ACK / Chk

3. MFD				                         Cycle/ Req data

4. Master Mode			                         NAV

# Base Leg Checks

1. Speedbrake					Extended

2. Gear 					Down

3. Airspeed				             ~200Knts

4. Wheel Brakes		                         Disengaged

6. Land				                         ILS + Land