START UP	 						1F-16C-1CL-1

1. Master mode					NAV

2. HUD				                          Data Present/Correct

3. Radio					Twr/Tcn Select

4. Radio					Chk Channel/Bckup Mode

5. A/C Lights					On

6. Master Arm/Safe				Auto

7. Controls					Full and Free

NOTE:

Beware of spending excessive time checking the aircraft. Always refer to the ACC. ** Max Taxi Speed 20 kn **

8. Speedbrakes 			             Incremental Inc/Dec

9. Throttle					Idle

10. Wheel brakes				Checked Locked

11. Throttle					Chk to full Mil Pwr

12. HUD				             Declutter/Scales Mode

13. HUD					Color / Panel Settings

14. U/C					             Check Locked (3xG)

15. Radar				             Set NO RAD. ACM.

16. DED					F-ACK / A-LOW (Chks)

17. MFD				             Cycle/ Data correct

18. Tower				             Request Taxi.

# Taxi

1. Speedbrake					Extended

2. Speed					< ~20Kn

3. Caution Panel				No Faults

4. Fuel/Flow				             Chk Correct

5. MPO					             Disengage

6. Wheel Brakes				Chk

7. Runway		                                       Hold Short

NOTE:

Excessive use of wheel brakes and/or differential braking is to be avoided.  Maximum safe taxi speed on ramps and congested areas: 20kn

USAF/DOD 11.2565.67			                                   F-16C-BL*50-52